DATA COMMUNICATION & NETWORKING(DCN)

SUBMITTED TO: DR. KIRTAN GOPAL PANDA SUBMITTED BY: IEC2023104

# GROUP: 20 GROUP MEMBERS:

- IEC2023102

- IEC2023103

- IEC2023104

- IEC2023105

- IEC2023106

## INDEX

| Sr. No. | Content | Page No. |
| --- | --- | --- |
| 1 | Cover Page | 1 |
| 2 | Index | 2 |
| 3 | Aim / Objective | 3 |
| 4 | Apparatus Required | 3 |
| 5 | Theory | 4 |
| 6 | Block Diagram / Network Setup | 6 |
| 7 | Procedure | 7 |
| 8 | Observation Table | 9 |
| 9 | Network Configuration Commands | 10 |
| 10 | Result | 11 |
| 11 | Applications | 11 |
| 12 | Conclusion | 12 |
| 13 | Precautions | 12 |

## AIM / OBJECTIVE

To configure a Raspberry Pi 5 as a dual-interface wireless router and hotspot by delegating the internal Broadcom Wi-Fi chipset as an Access Point (AP) and an external USB Wi-Fi adapter (TP-Link N150 – RTL8188 chipset) as the internet-receiving client interface, thereby creating a private sub-network for downstream wireless devices.

## APPARATUS REQUIRED

| S.No | Apparatus / Component | Quantity | Specification / Remarks |
| --- | --- | --- | --- |
| 1 | Raspberry Pi 5 (8GB RAM) | 1 | Primary computing unit / SBC |
| 2 | TP-Link N150 USB Wi-Fi Adapter (RTL8188) | 1 | External Wi-Fi (Client / Internet Ingress) |
| 3 | 32 GB Micro SD Card (Class 10) | 1 | OS storage medium |
| 4 | USB-C Power Supply (5.1V / 5A) | 1 | Power supply for Raspberry Pi 5 |
| 5 | HDMI Cable + Monitor | 1 | Display for initial configuration |
| 6 | RJ45 Ethernet Cable | 1 | Alternate internet source (wired LAN) |
| 7 | Laptop / PC | 1 | SSH / VNC validation, client throughput testing |
| 8 | Smartphone (Wi-Fi Client) | 1 | Wireless client for hotspot testing |
| 9 | Raspberry Pi OS Bookworm (64-bit) | 1 copy | Operating system (software) |
| 10 | Raspberry Pi Imager v1.7+ | 1 copy | OS flashing utility (software) |

## THEORY

### Development Boards and Raspberry Pi

A development board is a printed circuit board designed to provide an accessible platform for developers to prototype and iterate smart products. Common examples include Arduino Uno, Intel Galileo, and Raspberry Pi. Unlike a raw microcontroller circuit, a development board integrates essential peripherals such as memory, I/O headers, communication interfaces, and power regulation on a single compact PCB, eliminating the need for a custom hardware build during the prototyping stage.

The Raspberry Pi is a single-board computer (SBC) family developed by the Raspberry Pi Foundation. The Raspberry Pi 5, used in this experiment, is powered by the Broadcom BCM2712 SoC (Cortex-A76 64-bit quad-core at 2.4 GHz) and ships with 8 GB of LPDDR4X RAM. It natively supports 802.11 b/g/n/ac dual-band (2.4 GHz and 5 GHz) Wi-Fi, Bluetooth 5.0, Gigabit Ethernet, and a 40-pin GPIO header, making it ideal for network-centric IoT applications.

### Dual-Interface Networking Concept

A single Wi-Fi chipset typically cannot simultaneously maintain a client connection to an upstream router and broadcast an independent SSID (Access Point). This physical and logical constraint

necessitates a secondary Wi-Fi interface. In this experiment, two separate interfaces are used and assigned distinct network roles:

    - wlan0 (Internal Broadcom BCM2712): Assigned to Access Point (AP) Mode. This interface broadcasts the private hotspot SSID to downstream client devices.

    - wlan1 (External Realtek RTL8188 via TP-Link N150): Assigned to Station (STA) / Client Mode. This interface connects to the host (campus/hotel) router and brings internet into the Raspberry Pi.

### Network Address Translation (NAT)

NAT is a networking technique where a gateway device (here, the Raspberry Pi) translates private IP addresses of devices within its local subnet into its own single public IP address before forwarding traffic to an external network. This enables all devices on the 192.168.4.0/24 hotspot sub-network to access the internet through the single IP address that the upstream router assigned to wlan1. The main router only ever sees traffic originating from the Raspberry Pi's MAC address, effectively masking all downstream client devices.

### DHCP (Dynamic Host Configuration Protocol)

The Raspberry Pi, acting as a DHCP server on its hotspot interface (wlan0), dynamically assigns IP addresses from the pool 192.168.4.2 – 192.168.4.20 to all connecting Wi-Fi clients. This eliminates the need for manual static IP configuration on each downstream device. The DHCP server also provides the DNS server address and gateway information (192.168.4.1) to clients.

### Access Point (AP) Mode vs. Station (STA) Mode

A Wi-Fi chipset operating in Station Mode acts as a client that connects to an existing SSID. In Access Point Mode, the chipset broadcasts its own SSID and accepts client connections, functioning as a wireless router. Not all Wi-Fi chipsets support AP Mode; the Realtek RTL8188 (on the TP-Link N150) supports only STA mode, which is why the internal Broadcom chipset – which natively supports AP Mode – is used to host the hotspot.

### nmcli – Network Manager Command Line Interface

nmcli is the command-line interface for NetworkManager, the daemon responsible for managing network interfaces in Raspberry Pi OS Bookworm. Unlike the legacy hostapd + dnsmasq + dhcpcd stack (used in older Buster/Bullseye OS versions), the Bookworm release uses NetworkManager as the unified network management backend. The nmcli tool is used in this experiment to identify interface names, create hotspot connections, manage SSIDs/passwords, and configure persistent autostart behaviour.

### Proxy Configuration

In a campus or institutional network, internet access is often routed through an HTTP/HTTPS proxy server. To allow the Raspberry Pi to download packages and updates via apt, the proxy must be configured at both the system environment level and the apt package manager level. The proxy used  in  this  experiment  is  172.31.2.4  on  port  8080.  The  configuration  is  set  in

/etc/apt/apt.conf.d/10proxy and in the user's .profile for environment-level http_proxy and https_proxy variables.

## NETWORK SETUP / BLOCK DIAGRAM

The following ASCII topology diagram represents the complete network architecture of the experiment. The Raspberry Pi 5 acts as the central gateway between the campus router and the downstream wireless client devices.

(Downstream Clients via DHCP: 192.168.4.2 – 192.168.4.20)

+	+

| PC	|

| .4.4	|

+	+

+

+

| Phone |

| .4.3	|

|

+

| .4.2

+

+	+	+	+

| Laptop |

+

(Hotspot: WiFi 2 / WPA2)

|

|

NETWORK TOPOLOGY DIAGRAM

[Internet]

|

| (WAN)

+	+

| Router | 10.10.0.1 (Host / Campus Router)

+	+

| (LAN / Wi-Fi: sparklers 2G)

|

+	+

| Raspberry Pi 5	| wlan1 (RTL8188) --> Client to Host Router

| wlan1: DHCP IP	| wlan0 (Broadcom) --> Hotspot (AP Mode)

| wlan0: 192.168.4.1/24	|

+

Note: The diagram above should be redrawn neatly in the notebook using a ruler. Label each interface, IP address block, and connection type (wired / wireless) clearly.

#### Interface Role Summary:

- wlan0 (Internal) → Broadcasts hotspot SSID | IP: 192.168.4.1/24 | Mode: AP

- wlan1 (External) → Connects to host router | IP: DHCP from host | Mode: STA

- eth0	→ Optional wired uplink  | IP: 10.10.0.x/24  | Mode: Client

## PROCEDURE

### Phase A: OS Installation and Initial Setup

- Download Raspberry Pi OS (64-bit) Bookworm from the official Raspberry Pi downloads page (raspberrypi.org/downloads). Note: Legacy OS versions (Bullseye/Buster) use a different network stack and are incompatible with the nmcli method used in this experiment.

- Install Raspberry Pi Imager on a host computer (Windows/macOS/Linux). Open the Imager and configure the following headless settings by clicking the gear/settings icon:

  - Hostname: raspberrypi.local

  - Enable SSH with password authentication.

  - Set username (pi) and a secure password.

  - Pre-configure Wi-Fi credentials of the bootstrap SSID (the campus/host network) so the Pi connects automatically on first boot.

- Insert a 32 GB Micro SD card into the card reader. Select the SD card as the storage target in Raspberry Pi Imager and click Write. Wait for the flashing and verification to complete.

- Remove the SD card and insert it into the Raspberry Pi 5. Connect the TP-Link N150 USB Wi-Fi adapter to a USB 3.0 port. Apply power via the USB-C supply.

### Phase B: Proxy Configuration (Campus Network)

- SSH into the Raspberry Pi: ssh pi@raspberrypi.local

sudo nano /etc/apt/apt.conf.d/10proxy

Configure apt proxy by editing the proxy configuration file:

Add the following lines:

Acquire::http::Proxy "http://172.31.2.4:8080";

Acquire::https::Proxy "http://172.31.2.4:8080";

- Configure system-level environment proxy variables by editing the user profile:

sudo nano ~/.profile

Append:

export http_proxy=http://172.31.2.4:8080 export https_proxy=http://172.31.2.4:8080

export no_proxy="localhost,127.0.0.1"

- Apply changes: source ~/.profile

### Phase C: Interface Identification and Internet Connection

sudo apt update && sudo apt upgrade -y

nmcli device

Update the OS and verify both Wi-Fi interfaces are detected:

The output should list wlan0 (Broadcom, internal) and wlan1 (Realtek RTL8188, external USB adapter).

sudo nmcli device wifi connect 'sparklers 2G' password 'YourPassword' ifname wlan1

Via the NetworkManager GUI (if HDMI monitor is connected), or via CLI, connect wlan1 to the campus/host router SSID. This provides the internet feed to the Raspberry Pi.

- Verify that wlan1 has received a DHCP IP from the host router:

nmcli device show wlan1

### Phase D: Hotspot Creation on wlan0

Method 1 – GUI (Desktop mode via HDMI):

- Go to the Wi-Fi icon in the system tray → Advanced Options → Create Wireless Hotspot.

- Set the SSID to WiFi RPI and assign it to the wlan0 (Broadcom) interface. Leave security as WPA2 and set a password.

Method 2 – Terminal (nmcli):

sudo nmcli device wifi hotspot ssid 'WiFi 2' password 'sparklers123' ifname wlan0

Create the hotspot directly from the terminal:

- Verify the hotspot is active:

nmcli device

wlan0 should now show 'AP' role; wlan1 should show 'connected' to the host router.

### Phase E: Configuring Hotspot Persistence

nmcli connection show

Retrieve the UUID of the hotspot connection profile:

Note the UUID value next to the 'hotspot' profile name.

sudo nmcli connection modify [UUID] connection.autoconnect yes

sudo nmcli connection modify [UUID] connection.autoconnect-priority 100

Configure the hotspot to auto-start on every boot:

Setting priority to 100 ensures NetworkManager initializes wlan0 in AP mode during boot, preventing it from defaulting to client/search mode.

- Reboot the Raspberry Pi to verify persistence:

sudo reboot

- After rebooting, confirm the hotspot SSID (WiFi 2) is visible from a smartphone or laptop without any manual intervention.

### Phase F: Client Connection and Validation

- On a laptop or smartphone, scan for Wi-Fi networks. Connect to 'WiFi 2' using password 'sparklers123'.

- Verify the client received an IP in the 192.168.4.x range.

- Test internet access by opening a browser and navigating to google.com.

- Optionally, ping the Raspberry Pi gateway from the connected device: ping 192.168.4.1

- SSH into the Raspberry Pi from the wireless client: ssh pi@192.168.4.1

## OBSERVATION TABLE

### Table 1: Network Interface Status

| Interface | Role | IP Address Assigned | SSID / Network | Status |
| --- | --- | --- | --- | --- |
| wlan0 (Internal Broadcom) | Access Point (AP) | 192.168.4.1 / 24 | WiFi RPI / WiFi 2 | Active – Hotspot Broadcasting |
| wlan1 (Realtek RTL8188) | Client / STA | 192.168.x.x (DHCP from host) | sparklers 2G (Host Router) | Connected – Internet Ingress |
| eth0 (Ethernet) | Wired LAN | 10.10.0.2 (Static / DHCP) | N/A (Wired) | Connected – Alternate uplink |

### Table 2: Client Device Connectivity Validation

| S.No | Device | SSID Connected | IP Assigned | Ping (ms) | Internet Check | Result |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | MacBook (Laptop) | Wi-Fi 2 | 192.168.4.2 | 8ms avg | google.com reachable | Pass |
| 2 | Android Smartphone | Wi-Fi 2 | 192.168.4.3 | 12ms avg | Browser navigation successful | Pass |
| 3 | Windows PC | RpiNetwork | 192.168.4.4 | 10ms avg | SSH to 192.168.4.1 verified | Pass |

## NETWORK CONFIGURATION COMMAND REFERENCE

### Essential nmcli Commands

Identify all network interfaces and their modes:

nmcli device

List all saved connection profiles with UUIDs:

nmcli connection show

Connect wlan1 to a specific SSID:

sudo nmcli device wifi connect 'SSID_NAME' password 'PASSWORD' ifname wlan1

Create a WPA2 hotspot on wlan0:

sudo nmcli device wifi hotspot ssid 'WiFi 2' password 'sparklers123' ifname wlan0

Enable autoconnect for a profile (replace UUID accordingly):

sudo nmcli connection modify [UUID] connection.autoconnect yes

sudo nmcli connection modify [UUID] connection.autoconnect-priority 100

### Diagnostic Commands

Check IP address assignment on all interfaces:

ip addr show

Display wireless interface details:

iwconfig

ifconfig

Ping the gateway to test connectivity:

ping 192.168.4.1

Check CPU temperature (Raspberry Pi):

vcgencmd measure_temp

Check available disk space:

df -h

### Troubleshooting – If Hotspot Fails After Reboot

Execute the following recovery sequence:

sudo rfkill unblock wifi

sudo ip addr add 192.168.4.1/24 dev wlan0 sudo systemctl restart NetworkManager

nmcli device wifi hotspot ssid 'WiFi 2' password 'sparklers123' ifname wlan0

## RESULT

The Raspberry Pi 5 was successfully configured as a dual-interface wireless router and hotspot. The TP-Link N150 USB Wi-Fi adapter (wlan1 – RTL8188 chipset) established a stable client connection to the host campus router (sparklers 2G), providing the internet uplink. The internal Broadcom chipset (wlan0) broadcasted a private WPA2-secured hotspot (SSID: WiFi 2) with the password 'sparklers123' on the 192.168.4.0/24 sub-network.

Three downstream client devices (laptop, smartphone, and PC) successfully connected to the hotspot and obtained DHCP-assigned IP addresses in the range 192.168.4.2 – 192.168.4.4. Internet access was verified on all clients by successfully navigating to google.com. The hotspot

connection was confirmed to auto-start after a system reboot without any manual user intervention, validating the persistence configuration.

## APPLICATIONS

- Travel Router: Sharing a single wired or authenticated Wi-Fi connection (hotel/conference) among multiple personal devices without additional subscription costs.

- IoT Gateway: Serving as the central network hub for a cluster of IoT sensors and actuators in a smart home or factory floor deployment.

- MAC Address Masking: Circumventing single-device connection policies of institutional networks by presenting a single Raspberry Pi MAC address to the host router while supporting unlimited downstream devices.

- Network Penetration Testing Lab: Creating an isolated wireless test environment for ethical hacking and security research.

- Educational Network Lab: Demonstrating DNS, DHCP, NAT, and routing concepts in a controlled and cost-effective hardware environment.

- Disaster Recovery Networking: Rapidly deploying a field access point in emergency communications scenarios where infrastructure is unavailable.

- Campus Extension Networks: Extending institutional network coverage to areas with weak signal using a Raspberry Pi as a wireless repeater.

## CONCLUSION

This experiment demonstrated the practical application of dual-interface networking on the Raspberry Pi 5. By assigning the internal Broadcom chipset to Access Point mode and the external RTL8188 adapter to client/station mode, a fully functional wireless router was created without any specialized networking hardware. The use of NetworkManager's nmcli utility on Raspberry Pi OS Bookworm provided a straightforward, script-less configuration path that is significantly more reliable than the legacy hostapd/dnsmasq stack on older OS versions.

The experiment reinforced key networking concepts including NAT, DHCP, interface role allocation, SSID management, and persistence configuration via connection priority. The Raspberry Pi's ability to manage an independent IP sub-network (192.168.4.0/24) while routing to the host network underscores its versatility as an IoT and network infrastructure device. The MAC masking capability observed in this setup highlights an important practical security and policy consideration in enterprise and institutional network design.